# Tender text

# Motion detector theMura S180-100 2W UP WH

Order No. white: 		2060670

Features:

- Passive infra-red motion detector for wall mounting in flush-mounted box

- Cover in Theben design

- Automatic presence and brightness-dependent control for lighting in combination with a staircase light timer switch from Theben (only 4-conductor switching)

- Rectangular detection area 170°, max. 14 m x 17 m

- Area restriction in scope of delivery

- Mixed light measurement suitable for fluorescent lamps (FL/PL/ESL), halogen/incandescent lamps and LEDs

- Integrated button (can be deactivated)

- Adjustable brightness switching value, teach-in function

- Pulse function for staircase light timer switch, time delay is set at the staircase light timer switch

- Ready for immediate use due to factory presetting

- Test mode for checking the detection area

- Extension of detection area via parallel switching

Technical data:

Operating voltage: 		230 V AC

Frequency: 			50 – 60 Hz

Power consumption: 		approx. 0.4 W

Radial detection area: 		12 x 10 m

Tangential detection area: 	14 x 17 m

Installation height Mh: 		0.8 – 1.2 m

Brightness switching value: 	5 – 1000 lux

Lighting time delay: 		pulse, time delay is set at the staircase light timer switch

Protection rating: 		IP20

Operating temperature: 		-15 °C to +45 °C

Dimensions: 			85 x 85 mm, installation depth 37 mm

Installation: 			wall mounting in flush-mounted box size 1

Optional accessories:

### ELPA 1

### Staircase light timer switch, electronic

Order No.:			0010002

### ELPA 3

### Staircase light timer switch, electronic

Order No.:			0030002

### ELPA 6

### Staircase light timer switch, electronic

Order No.:			0060002

### ELPA 6 plus

### Staircase light timer switch, electronic

Order No.:			0060003

### ELPA 8

### Staircase light timer switch, electro-mechanical

Order No.:			0080002

### ELPA 041

### Flush-mounted staircase light timer switch, electronic

Order No.:			0410002

### ELPA 047

### Flush-mounted staircase light timer switch, electronic

Order No.:			0470002