2-wire motion detector for flush-mounted wall installation, light beam, 200°, max. R=8m, IP40 (LUXA 103-200 T)

Add-on motion detector for flush-mounted wall installation, 200° detection area, adjustable range 1-8 m radius (installation height 1.2... 2 m), detection area can be limited using plug-in segments, can be used as replacement for light switch or changeover switch, suitable for 50x50 mm cover, integrated ON/AUTO/OFF manual switch, brightness and movement-dependent automatic switching, blended light measurement approx.10...1000 lx, power-on time 5 s...12 min, triac output light: 2 A/230V~, 25-400 W incandescent lamp load, white 80 x 80 mm frame, with terminal screws.

Deliver, assemble, connect up and set up

Make: Theben

Type: LUXA 103-200 T

Order no.: 103 0 031

# Unit: ST

Accessories:

## RC link suppression filter

For protection against damage from power surges or switch-on (contacts) of detection sensor caused by contactors, fluorescent lamps, transformers.

250 V AC/47 Ohm, 0.1 µF

Dimensions: 24 x13 x 6mm

Order no.: 9070523