Rail mounted 7 day timer, segments, power reserve, 1 TLE (MEM 190 a)

Rail mounted timer with approx. 3 days power reserve; width 17.5 mm; 7 day segment program; shortest switching time 2 hours; permanent ON/OFF switching; 1 NO contact 16A/250V~; sealable transparent cover.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: MEM 190 a

Order No.: 1900001

Unit: ST