Presence sensor for wall installation, maximum 7x7 m (Presence Light 180)

Theben HTS presence sensor for wall installation, fully automatic light control, blended light measurement  10...1500 lx, 1 light relay, maximum range 7x7m at installation height of 3 m, light relay: 1 NO contact 6 A 1400 VA, delay 10 seconds...20 minutes auto-learning / short pulse, protection class IP54, can be remotely operated, front plate insulation material white 86x86x27 mm, mounting plate 70x70 mm, installation depth 40 mm

Deliver, assemble, connect up and set up

...

Make: Theben

Type: Presence Light 180

Order No.: 200 0 050

Unit: ST

Accessories:

# QuickSet plus

Service remote control

Order No.: 9070532

Clik

User remote control

Order No.: 9070515

# AP frame Presence Light, white

Order No.: 9070513