EIB/KNX thermostat for fan coil- continuous control for 2 or 4 line-systems (RAM 713 FC EIB/KNX)

EIB/KNX thermostat for fan coil actuators; installation housing 84 x 80 x 27 mm; colour: white; continuous or 2 point individual room control with integral bus coupler; setting range 10-28°C;

Applications: Fan coil; heating and cooling with fan coil. Manual switch for selection of fan stages Off/Stage 1/Stage 2/Stage 3/Automatic 3 binary inputs for conventional keys with the following functions: switching, dimming, blinds Inputs can also be used for external temperature sensors, window contact or presence sensor.

Deliver, assemble, connect up and set up

Make: Theben

Type: RAM 713 EIB/KNX

Order No.: 7139202

Unit: ST

Accessories:

External temperature sensor

Order No.: 907 0 321