Clock thermostat with radio control, 7 day, 1 channel DIN rail receiver (RAM 813 top 2 HF Set 1)

Elegantly designed digital room thermostat with radio control; info button for important information; text line in the display with step-by-step operator guide; weekly program; 3 basic programs with modifiable temperatures and an individual program with up to 42 time/temperature phases; setting range +6°C ... 30°C in 0.2 degree increments; date-controlled holiday program; fully automatic summer/winter time changeover; party and ECO program with adjustable duration; frost protection a summer switching; operating point and switching differential can be adjusted. Power supply via two 1.5 V AA alkaline batteries. Low battery indication. Housing colours: white. Coded switching commands are transmitted by radio (868MHz) to a mains-operated receiver, e.g. in a boiler room. For wall mounting or DIN rail installation (45 x 90 x 68 mm). Range indoors approx. 25-30 m; with HF level indication. Transmitter and receiver factory pre-coded. Receiver with 1 two way switch 6 A/250 V~.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RAM 813 top 2 HF Set 1

Order No.: 8139501

Unit: ST