KNX 4-way switch actuator, upgrade device (RME 4 I KNX)

KNX 4-way C load switching actuator with relay outputs for 4 switching channels; upgradeable with RME 4I for up to 12 channels; rail mounting housing width 72 mm; LED switching status display for each channel; manual switch with status display (also without bus voltage); functions: Switching, delayed switching impulse, links, type of contact (NC contact/NO contact) and participation in central commands such as continuous On, continuous Off, central switching and save/call up scene; switching functions: On/Off, impulse, On/Off delay, staircase light with warning according to DIN 18015-2; logical links: lock, AND, release, OR; features: Selectable response in event of bus failure as well as in the event of return of bus/mains voltage, feedback object for each channel, central objects with and without priority, select and save scenes (object); scene object for every channel; switching capacity: 16A/250V~; cos phi= 0.6;

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RMG 4 I KNX

Order no.: 4930215

Unit: ST