KNX 4-way switch actuator, basic device (RMG 4 I KNX)

KNX 4-way C load switching actuator with relay outputs for 4 switching channels; upgradeable with RME 4I for up to 12 channels; rail mounting housing width 72 mm; basic device and bus module can be separated mechanically and can be swapped independently of each other. LED switching status display on device for each channel; manual switch with status display (also without bus voltage); links, type of contact (NC contact/NO contact) and participation in central commands such as continuous On, continuous Off, central switching and save/call up scene; switching functions: On/Off, impulse, On/Off delay, staircase light with warning according to DIN 18015-2; logical links: Lock, AND, release, OR; feedback object for each channel, central objects with and without priority, select and save scenes (object); scene object for every channel; switching capacity: 16(10)A/250V~

Deliver, assemble, connect up and set up

...

Make: Theben

Type: RMG 4 I KNX

Order no.: 4930210

Unit: ST