KNX control unit 1 - 10 V, 2-way, basic device (SMG 2 S KNX)

KNX  2 channel control unit combined with electronic upstream devices (EVG´s) enable switching and dimming of lighting circuits; upgradeable with SME 2 S for up to 6 channels or can be used in combination with other MIX series devices; rail mounted housing width 72 mm; 2 relay switching outputs 16 A / AC-1 ; 10A/ AC –3; nominal voltage 230V with zero cross switching as well as one 1-10 V output per channel; characteristics: Also suitable for dimmable energy-saving lamps; logical diagnosis and feedback objects for each channel; manual switch for ON, OFF, BUS even without bus voltage.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: SMG 2 S KNX

Order no.: 4910273

Unit: ST