# Tender text for motion detector

# Motion detector theMova S360-100 UP WH / GR

Passive-infrared motion detector for ceiling installation; automatic motion and brightness-dependent control for lighting; round 360° detection area, walking up to 452 m2 (Ø 24 m), installation height up to 10 m, area restriction with covering clips (optional);

Mixed light measurement suitable for fluorescent lamps (FL/PL/ESL), halogen/incandescent lamps and LEDs;

Operating voltage 110 - 230 V AC / 50 - 60 Hz; energy consumption approx. 0.1 W, Protection rating IP 20, in fitted state IP 40; operating temperature -15° C to +50° C;

Channel A light: relay, 1 NO contact 230 V / 10 A, 2300 W or 1150 VA (cos ϕ = 0.5); fully automatic operation; configurable brightness switching value approx. 30 – 3000 Lux, teach-in function; time delay 10 s – 60 min; pulse function for staircase light timer switch;

Configurable sensitivity; ready for operation immediately thanks to factory presetting; can be configured remotely with remote control; test mode to check function and detection area;

Installation depth 37 mm, ceiling installation in flush-mounted box, size 1 or ceiling installation socket 73A, ceiling installation possible with back box 110 A, Protection rating IP 44;

Manufacturer: thebenHTS

Type: theMova S360-100 UP.

Order no.: 1030600 (theMova S360-100 UP WH, white)

1030601 (theMova S360-100 UP GR, grey)

Unit: ST

### Accessories

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order No.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911

### Back box 110A WH

Optional

Order no.: 9070912

### Back box 110A GR

Optional

Order no.: 9070913

### Ceiling installation socket 73A

Optional

Drill diameter 73 mm

Order no.: 9070917

### Covering clip for area restriction

Optional

Order no.: 9070921