# Tender text for presence detectors

# Presence detector thePrema P360 Slave E UP WH / GR

Passive-infrared presence detector for ceiling installation; square 360° detection area, walking 100 m2 (10 x 10 m), seated 64 m2 (8 x 8 m) at 3.5 m installation height, installation height up to 10 m;

Operating voltage 110 - 230 V AC / 50 - 60 Hz, energy consumption approx. 0.1 W; Protection rating IP 20, in fitted state IP 40;

Configurable sensitivity; ready for operation immediately thanks to factory presets; can be configured remotely with remote control; test mode to check function and detection area; extension of detection area by master/slave or master/master switch;

Installation depth 40 mm, ceiling installation in flush-mounted box size 1 or ceiling installation socket 73A, ceiling installation possible with back box 110A;

Manufacturer: thebenHTS

Type: thePrema P360 Slave E UP.. (Can only be used with a master device)

Order no.: 2070130 (thePrema P360 Slave E UP WH, white)

2070131 (thePrema P360 Slave E UP GR, grey)

Unit: ST

### Accessories

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order no.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### Back box 110A WH

Optional

Order no.: 9070912

### Back box 110A GR

Optional

Order no.: 9070913

### Ceiling installation socket 73A

Optional

Drill diameter 73 mm

Order no.: 9070917