# Tender text for presence detectors

# Presence detector theRonda P360-100 M UP WH / GR

Passive-infrared presence detector for ceiling installation; automatic presence and brightness-dependent control for lighting; round 360° detection area, walking up to 452m2 (Ø 24m), installation height up to 10m, area restriction with covering clips (optional);

Mixed light measurement suitable for fluorescent lamps (FL/PL/ESL), halogen/incandescent lamps and LEDs;

Operating voltage 110 - 230 V AC / 50 - 60 Hz; energy consumption approx. 0.1 W, protection rating IP 20, in fitted state IP 54; operating temperature -15°C to +50°C;

Channel A light: relay, 1 NO contact 230 V / 10 A, 2300 W or 1150 VA (cos ϕ = 0.5); can be switched between fully automatic or semi-automatic operation; configurable brightness switching value approx. 30 – 3000 Lux, teach-in function; time delay 10sec. – 60min.; reduction of the time delay when present briefly (short-term presence); connection option for push button or switch for manual switching with automatic detection push button/switch; pulse function for staircase light timer switch;

Configurable sensitivity; ready for operation immediately thanks to factory presetting; can be configured remotely with remote control; test mode to check function and detection area; extension of detection area by master/slave or master/master switch;

Installation depth 37mm, ceiling installation in flush-mounted box size 1 or ceiling installation socket 73A, ceiling installation possible with back box 110A;

Manufacturer: thebenHTS

Type: theRonda P360-100  M UP.

Order no.: 2080020 (theRonda P360-100 M UP WH, White)

2080021 (theRonda P360-100 M UP GR, Grey)

Unit: ST

### Accessories

Extension of detection area via slave device:

Optional

Type: theRonda P360 Slave UP..

Order no.: 2080030 (theRonda P360 Slave UP WH, White)

2080031 (theRonda P360 Slave UP GR, Grey)

### Management remote control "SendoPro 868-A"

Optional

For the installer for start-up or adjustment to usage conditions.

Text-based user guidance in the display. Transmission of individual setting values or complete parameter sets to the detector possible. Suitable for all remote controlled thebenHTS presence detectors.

Order No.: 9070675

### Service remote control "theSenda P"

Optional

Order no.: 9070910

### User remote control "theSenda S"

Optional

Order no.: 9070911

### Back box 110A WH

Optional

Order no.: 9070912

### Back box 110A GR

Optional

Order no.: 9070913

### Ceiling installation socket 73A

Optional

Drill diameter 73mm

Order no.: 9070917

### Cover clip for area restriction

Optional

Order no.: 9070921