Rail mounted digital time switch 1 channel,  7 day,  plug-in terminals, 1 module (TR 608 top2 S)

Digital 1 channel time switch with weekly program and 56 memory locations; rail mounted device with transparent cover and quick fastening; width 17,5 mm (1 module); text-based operator guidance in the display; interface for OBELISK top2 memory card for PC programming; 10 year lithium battery power reserve; ON/OFF switching times; switching preselection; permanent ON/OFF switching; integrated operating hours counter; holiday program; PIN code; automatic summer/winter time changeover, change-over contact 16(4)A/250VAC, operating voltage 230-240V/50-60 Hz; stand-by 0.4 W; DuoFix spring terminals, high accuracy due to temperature compensation.

Delivery, installation, connect and set up.

Make: Theben

Type: TR 608 top2 S

Order no.: 6080101

Unit: ST

Accessories:

OBELISK top 2 programming set

Memory card, plug adapter, software CD, USB plug adapter, for Windows 2000/XP/Vista.

Order no.: 9070409

OBELISK top2 memory card, single

Order no.: 9070404

17.2.2014