KNX display with multifunction, individual room control and time switch (VARIA 826 S WH KNX)

KNX display with multifunction in extremely flat design; flush-mounted installation; panel 128 x 93 x 18 mm; colour white, glass front; display with 7 different illuminated display pages each with 8 lines; pre-programmed for temperature and weather, 5 freely programmable pages e.g. for lighting, blinds, ventilation etc.; display and manual control of 40 functions available via keypad; weekly timer with maximum of 8 channels, continuous individual room control with integrated bus coupling unit; setting range 10-28°C; applications: Heating via radiator or under floor heating; 2-step heating with basic and additional stages (under floor and radiator heating); heating and cooling, e.g. radiator and cooling surface; comfort, standby, night-time temperature reduction and frost protection operation modes.

Deliver, assemble, connect up and set up

...

Make: Theben

Type: VARIA 826 S WH KNX

Order no.: 8269210

Unit: ST